Lab Activity No. _6_

# Basic Router Configuration

Yr./Sec. : II- EINS	 Course: 	Networking 1			Date Started : 10/10/2026		 Grp. No. :  4						Date Finished : 	  Names :  Yabut, Renyer N. Utrera, Sean John Daniel

# OBJECTIVES:

  - To configure internal interface of a router

  - To configure SSH and MOTD

  - Configure the following in the Routers of Makati.

    - Password

    - Secret	d.	Console

c.	VTY	e.     SSH

  - Access router using VTY

# MATERIALS:

  - Working PC

  - Cisco Simulator software

  - Lab. Activity No.6 data Sheet

  - Diagram/Figure 6

# PROCEDURES:

  - Using packet Tracer, construct the diagram in figure 6.

  - Configure the router using the Command given below.

  - Change necessary details pertaining to your group no. , motd and passwords

  - Follow label in each device. (Admin Laptop, Makati Router, Sw1, Terminal 1)

  - You are not allowed to directly open the router from its icon.

  - Configure router using only console terminal (Admin Laptop)

  - Configure router using only CLI

  - Configure Hostname = Makati

  - Console password = cisco1

  - Secret Password = cisco2

  - VTY Password = cisco3

  - Assign the first 5 VTY terminals for Telnet

  - Password for virtual terminal = cisco3

  - Assign the last 11 VTY terminals for SSH

  - Encrypt communication using SSH with 1024 bits key

  - Configure SSH (SSH username and password: netw1 secret cisco4

  - Configure MOTD = Group No._Activity No.6 (ex. Group No.1 Activity No.6)

  - Check router configuration in Terminal 1 using VTY. Access only in Command Prompt

    - Access Makati Router Using Telnet

    - Makati Router Using SSH

  - Save your work as your group number and activity number (ex. Grp1_Act6)

# Sample Configuration:

Configure interface on the router Makati#conf t Makati(config)#int fa 0/0

Makati(config)#ip address 192.168.10.1 255.255.255.0 Makati(config)#no shutdown

Makati(config)#^Z Makati#

Makati#cop r st or wr

Destination filename [startup-config]? Building configuration...

[OK]

# Configure interface on Terminal 1

Open Terminal 1, go to desktop, then select IP Configuration Configure the ff.

IP address = 192.168.10.2 Subnet mask = 255.255.255.0 Gateway = 192.168.10.1

# Test Connection

Makati# ping 192.168.10.2

>\ping 192.168.10.1

# Internal Configuration

Router#Conf t

Enter configuration commands, one per line. End with CNTL/Z. Router(config)#hostname Makati

Makati(config)#banner motd " Group No.	Activity No.6" Makati(config)#

Makati(config)#enable password cisco1

Makati(config)#enable secret cisco2

Makati(config)#line console

Makati(config-line)#

Makati(config-line)#password cisco3

Makati(config-line)#login Makati(config-line)#exit Makati(config)#

Makati(config)#line vty 0 4 Makati(config-line)#password cisco4 Makati(config-line)#login

Makati(config-line)#service password-encryption

Makati(config)#

Makati#

Makati#copy running-config startup-config (cop r st) wr

Makati(config)#ip domain-name netw1.com

Makati(config)#crypto key generate rsa How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK]

Makati(config)#username netw1 secret cisco5

Makati(config)#line vty 5 15

Makati(config-line)#login local Makati(config-line)#transport input ssh

Makati(config-line)#^Z

Makati#

# Checking:

## To check your configuration

Router#show running-config

## To check successful configuration

  - Open Admin Console must have now its password

  - Open terminal 1

  - Go to desktop command prompt

  - Type the ff.

    - telnet 192.168.10.1	(access router remotely using telnet)

    - ssh -l netw1 192.168.10.1	(access router remotely using ssh)

- Acquired Skills and Knowledge